ROV Testing Methods:

Camera Testing

Product:

- For the 4k camera, we need to test if it is capable of transmitting the large amount of data feed from it given the high quality output of it.

  - Plan: Test the 4k camera by calculating the bits per second of the 4k camera and check microcontroller and tether transfer rate

  - C&C 5.2.1b, C&C 5.2.1c, C&C 5.2.3

- Ensure that the brio100 can connect to the monitor through the raspberry pi

  - Plan: Test the brio100 by connecting to the the Pi and connecting the Pi to the monitor

  - C&C 5.2.2, C&C 5.2.4

- Ensure that the cameras are capable of seeing in the underwater conditions

  - Plan: Put the cameras within their underwater housing and put them in a bath, cover the bath with a sheet and see how well the camera+light are able to output video to the monitor/picture to the Pi, do this with the photogrammetry camera and navigational camera

  - C&C 5.2.3, C&C 5.2.1a, C&C 5.2.1b, C&C 5.4.1

Microcontroller Testing

Product:

- Ensure that the microcontroller is able to handle all the data that will run through it.

Temperature Sensor Testing

Product:

- The temperature capabilities of it will need to be tested along with the accuracy.

  - Plan: Dunk it into a cool tub of dry ice. We can test the temperature it outputs against a traditional thermometer.

Light Testing

Product:

- The lights will need to be tested alongside the camera to ensure it can provide clarity underwater for the cameras.

  - Plan: Test different setups of the light in relation to the camera to see which provides the best clarity for the camera in dark environments underwater.

- Temperature rating testing

  - Plan: Dunk it into a cool tub of dry ice.

- Light changing testing

  - Plan: Ask EPS before testing about possibly sending lower current to the lights to dim when needed.

Depth Sensor Testing

Product:

- Temperature rating testing

  - Plan: Connect the depth sensor to the raspberry pi 5 and connect the microcontroller to a computer to view output of the depth sensor. To reach as close to -25 degrees Celsius, dunk it into a cool tub of dry ice. Dunk the Depth Sensor itself into the water and see if it can output the temperature.

- Accuracy testing

  - Plan: Use a thermometer (such as a laser thermometer meant for cold temperatures) to compare and contrast what the depth sensor says vs the thermometer.

C3 Station Testing

Product:

- The laptop monitor will likely output in 1080p and the camera records in 1080p. Ensure the camera feed can properly be displayed to the laptop

  - Plan: Check the pilot view on the laptop and ensure that the vision is clear and not having issues with the feed.

- Ensure the Logitech controller can properly pilot the ROV from the C3 station

  - Plan: Connect the controller to the raspberry pi in the C3 station and ensure all our button commands are working properly

Float Testing Methods:

Product:

- Check the accuracy of depth reading on depth sensor

  - Plan: Use a tub of water to have the depth sensor read data in and constantly measure the true depth of the sensor as we move it up and down through the tub

- Check reliability of data transfer over bluetooth

  - Plan: Have a test run with the bluetooth to raspberry pi and ensure the data reads correctly, matching the provided format we need to have our depth data in