Requirements:

- Prior to competition, the float must complete a vertical profile of a depth of 2.5 meters, maintaining that depth for 30 seconds. Then ascend to a depth of 30cm and hold that depth for 30 seconds.

  - Once the float has been deployed, it must communicate to the receiver located on the surface at the shore station prior to descending by sending at least ONE data packet. This applies to the competition itself too

- During the vertical profiles:

  - Maintain a depth of 2.5 meters (+/- 33 centimeters) for 30 seconds

  - Ascend to a depth of 40 centimeters (+/- 33 centimeters) without breaking surface or contacting ice sheet for 30 seconds.

  - If the float drifts outside of this range at any time during the 30 seconds, the float must return to the designated range for an entirely new 30 second period.

    - For example, if the float maintains depth for 20 seconds but then goes outside of the given range, the float must ascend back into the range for an entire 30 seconds, not just the remaining 10 seconds from the first attempt at maintaining depth.

- After recovery and transmission:

  - the data packets should show seven (7) sequential data packets where the depth is 2.27 meters to 2.83 meters. Display all 7 data packets on the screen.

  - the data packets should show seven (7) sequential data packets where the depth is 0.07 meters to 0.73 meters. Display all 7 data packets on the screen

  - Companies must graph the data received from their vertical profiles, and there must be at least 20 data packets included on the graph.

- Next

Info to Consider:

- The bottom of the float should be used for calculating the 2.5 meter depth, i.e., the bottom of the float should be at 2.5 meters (+/- 33 cm).

- The top of the float should be used for calculating the 40 centimeter depth.

- If the float’s depth/pressure sensor is not at the bottom of the float, communicate the offset to the station judge.

  - For example, if the float’s depth/pressure sensor is 25 cm above the bottom of the float, when the bottom of the float is at 2.5 meters, the pressure sensor would be at 2.25 meters. Thus, the proper range for the depth/pressure sensor would be 2.58 meters to 1.92 meters. Communicate that adjusted range to the judge prior to deployment.

- There must be seven sequential data packets spanning 30 seconds (0, 5, 10, 15, 20, 25 and 30)

- Next

Functions we potentially need still:

- Function to direct the buoyancy engine to go down when detecting ice; actuate_buoyancy_engine()?

- Function for logic of direction of buoyancy engine to go?

- Function to send signals to the computer after recovery;

- Function to properly format data into packet;

  - [EX01 1:51:42 UTC 9.8 kpa 1.00 meters]

- Function to send data to the sd card?

https://docs.micropython.org/en/v1.20.0/mimxrt/quickref.html

https://github.com/bluerobotics/ms5837-python

Concerning bluetooth:

- https://www.raspberrypi.com/documentation/microcontrollers/

- https://pip-assets.raspberrypi.com/categories/686-raspberry-pi-pico-w/documents/RP-008257-DS-1-connecting-to-the-internet-with-pico-w.pdf?disposition=inline

- https://www.raspberrypi.com/news/new-functionality-bluetooth-for-pico-w/?utm_source=chatgpt.com

- https://github.com/micropython/micropython-lib/tree/master/micropython/bluetooth/aioble

- https://docs.micropython.org/en/v1.20.0/library/bluetooth.html?highlight=bluetooth

- Python bleak documentation

  - https://pypi.org/project/bleak/

  - https://bleak.readthedocs.io/en/latest/

  - https://docs.python.org/3/library/asyncio.html