# Entire

# C3 Station

ROV Pilot:

- The ROV pilot provides instruction and task to the Pilot-Computer Interaction Module

Monitoring Unit Information:

- Laptop

  - Powered by the AC power outlet provided.

  - Connects from ethernet cable port to the tether

Pilot-Computer Interaction Module

- Logitech controller

  - Will plug in to USB‑A port on C3 Station

ROV State Display Module

- IMU Information

- Stuff

Power:

- Wall outlet will connect to

# Tether:

Ethernet Cable

- Cat 5e Lan Cable

- Link

Power cable

- Link

# ROV (Internal Components)

- Thermistors

- Blue Robotics IMU Navigator

  - (For both power and data it will specifically connect to the Raspberry Pi 5’s 40-pin GPIO header “J8”. Align pin 1 on the Navigator with pin 1 on the Pi 5 header (pin 1 is the 3.3 V corner nearest the USB‑C power jack))

- Raspberry Pi 5 will have the 5v power/ground line connected through the USB C port

Power:

- Printed Circuit Boards

  - (Connect each ESC signal wire to a Pico PWM GPIO like GP0 pin 1 or GP1 pin 2 and tie ESC ground to a Pico GND like pin 3. Provide 5V from the ESC BEC to Pico VSYS pin 39 if you want the ESC to power the Pico or leave VSYS unpowered and power the Pico by USB VBUS pin 40. Ensure all grounds are common and send standard 1 to 2 ms servo pulses.)

- 12V to 5V transformer(Will power Raspberry Pi 5)

  - Will be connected to Raspberry Pi 5, specifically use a 12V to 5V DC converter. Connect its 5V output to the Raspberry Pi 5 USB C power port or to the 40 pin header 5V on pin 2 or pin 4 and GND on pin 6 or pin 9 or any GND. Ensure the converter supplies 5V 5A and all grounds are common.

- 5V to 3.3V transformer (Will power Raspberry Pico W)

  - Will be connected to the regulator input to Raspberry Pi 5V on pin 2 or pin 4 and GND to any ground such as pin 6. Send the regulator 3.3V output to your 3.3V peripherals only and keep grounds common. Do not backfeed the Pi 3V3 pins with that output.

Data:

- stuff

# ROV (External Components)

Power:

- Arducam 4K Camera

  - The power will be provided through the Raspberry Pi 5.

- Brio 100 Camera

  - Powering the Brio 100 Camera would be through

- Lights

  - The lights will be connected to a Relay to the Raspberry Pi 5 to allow for turning the lights on and off.

- Diamond Dynamics Thrusters

  - Will be connected to port Microcontroller (Raspberry Pico W) specifically each ESC signal wire should be connected to a PWM capable GPIO on the Raspberry Pi Pico such as GP2 GP3 GP4 or GP5.

  - Tie the ESC ground to a Pico GND pin and share ground with the battery.

  - Feed battery voltage only to the ESC power leads and do not power the ESC or thrusters from the Pico 3V3 or VSYS pins.

- Hobby servos

Data:

- Arducam 4K Camera:

  - The Arducam 4K camera will be connected to the USB 2.0 port of the Raspberry Pi 5.

- Brio 100 Camera:

  - will be connected to the USB 2.0 port of the Raspberry Pi 5.

- Hobby Servos

  - Link

  - Used to control motion of Claw. It will be connected to the Raspberry Pi 5 GPIO Pins

- Claw

  - Claw will consist of 4 servos

- Stuff