## Power Input (Tether)

- Tether (power+control+video): This is the physical cable connecting the Topsides to the underwater ROV. It carries all necessary power, control signals, and video/data between the two systems.

- SubConn Penetrator: A specialized, high-pressure, water-tight connector that allows the tether to pass through the ROV's hull while maintaining a seal.

- 48V Main Bus: The primary power distribution point on the ROV. It takes the main 48V supply from the tether (via the penetrator) and distributes it to the different subsystems.

ROV Subsystems (48V Main Bus Outputs)

- 48V-12V Motor Rail: A power path dedicated to the high-power components that drive the ROV.

  - Electronic Speed Controllers (ESCs): Devices that regulate the speed and direction of the thrusters based on control signals from the Raspberry Pi.

  - Thrusters: The underwater propellers that move and steer the ROV.

- 48V-5V Logic & Peripherals: A power converter that steps down the main 48V power to 5V, which is the common voltage needed for digital logic, microcontrollers, and many sensors.

  - Servos, Payload: Includes motors for manipulating arms/tools (servos) and any mission-specific equipment (payload) that require power.

  - Raspberry Pi (PI) (Ctrl & Telemetry): The main onboard computer. It processes control inputs, executes ROV logic, collects sensor data, and manages the telemetry (data sent back to the Topsides).

- Sensor Converters (3.3V/5V): Power converters that may step down the voltage further to supply specific sensors that require 3.3V or other regulated voltages.

  - IMU / Depth: The Inertial Measurement Unit (IMU) tracks the ROV's orientation (pitch, roll, yaw), and the Depth sensor measures the pressure to determine depth. These send data back to the Raspberry Pi.

- Power Monitor: A sensor system that measures the voltage, current, and potentially power consumption of the ROV's subsystems. This data is critical for monitoring the health of the system.

## Data Flow

- Telemetry to Topsides: The data path for sending information (sensor readings, status, video) from the Raspberry Pi back up the tether to the surface. This is the communication link.

Topsides Subsystems

- 48V Supply: The main power source for the entire system, providing the 48V DC that runs down the tether.

- Breaker / E-stop: A safety circuit that allows for manual or automatic interruption of the power supply. The E-stop (Emergency Stop) provides an immediate way to cut all power in an emergency.

- Primary Fuse: A sacrificial component designed to protect the entire system by blowing (opening the circuit) if the current exceeds a safe limit (e.g., due to a short circuit on the tether or ROV).

- Powerpole: A common type of high-current connector used for reliable, safe power distribution.

- Kill Relay / Solid State Relay (SSR): A safety feature on the ROV side that allows the Topsides to remotely cut power to the thrusters and high-power components without interrupting the control/telemetry link. This is a crucial safety "kill switch."