Detailed Breakdown of Propeller Classification:

- Pitch: theoretical distance a propellor moves forward in one revolution

  - TYPES:

    - Fixed-Pitch Propeller (FPP): blade angles permanently set in fixed position

    - Controllable-Pitch Propeller (CPP): Allows blades to rotate around their axis adjusting the pitch. Allows change in speed and direction without reversing the motors.

    - Folding Propeller: when motors not running, blades fold backward to reduce drag

    - Feathering Propellor: Blades rotate to align with the flow of water, even less drag than a folding propeller.

- Blade Number: affects speed, thrust and vibration:

  - TYPES:

    - Two-Bladed: high efficient, but more vibration

    - Three-Bladed: balance of speed and efficiency

    - Four-Bladed: stronger acceleration and better grip, but top speed decreased

    - 5 or More: very smooth ,quiet, excellent thrust.

  - How many propeller blades do you need?

  - Evolution of Propellers and Thrust Equation

- Blade Shape: different needed for different performance needs

  - TYPES:

    - Conventional: rounded blades

    - Cupped: small, curved lip on blade’s trailing edge to increase thrust and reduce slippage

    - Cleaver: high speed design, straight blades for surface piercing applications

    - Weedless: high skew to shed weeds and debris in contaminated/dirty water

- Material: effective durability, cost, performance. Specific heats should be considered so that the propellers do not deform.

  - TYPES:

    - Aluminum: common, affordable, lightweight but slightly less durable than other options

    - Stainless Steel: more durable and stronger than aluminum, can use thinner blades

    - Bronze: strong, corrosion resistant, rigid.

Common Selection Data:

- Larger diameter = lower RPM = more thrust per watt

- More blades = smoother thrust and less cavitation but higher drag

- Higher pitch = higher speed for a given RPM but might cause motor overload

- Shrouded propellers give higher static thrust for same wattage

Information Needed from the ROV:

- Pick target thrust per thruster based on vehicle mass & tasks

  - Propeller performance charts provided to map thrust -> power

- Propellor needs to match to motor (in voltage, kV, and torque)

Notes for designing a propellor:

- NACA 66 (mod) [General marine use, industry standard]

- DTMB 4119 [submarine propellers, US Navy Standard]

- Eppler 800 series [small scale hydrofoils, smooth pressure distribution]

- YS-900 series [advanced submarine/torpedo]

For our smaller application, the best choice would most likely be the Eppler 800 series

Through Picking a Cross Section, Some design points are set in stone, while others are variable by design:

- Static Parameters (defined by the Hydrofoil)

  - Camber line shape (determines pressure distribution and lift curve)

  - Maximum Camber (%)

  - Thickness Distribution

  - Leading Edge radius

- Adjustable Design Parameters:

  - Number of Blades (fewer blades = higher efficiency, more blades = smoother and quite flow but more drag)

  - Diameter (bigger diameter = more thrust)

  - Pitch/Pitch Distribution (ratio of advance per revolution)

  - Blade Area Ratio

  - Radial Hydrofoil Variation (can mix foil sections along the span)

  - Hub Ratio (Hub Diameter)

  - Skew and Rake (3D shaping of blades)

Reynold’s Number Calculation

Re =  ρVLμ = (1000kg/m3)(10m/s)(0.05m)(10-3kg/m*s) = 500,000

Calculations using:

- Density of water

- Viscosity of Water

- Top expected velocity

- Maximum expected length from leading to trailing edge of propeller cross section

ROV Design and Customization

Research Papers:

Aspects of Propeller Developments for a Submarine, By Anderson

Anderson’s Design Spiral for Submarine Propellers:

- Start Propeller Design

- Hydrodynamic

- Hydroacoustic

- Structural Analysis

- Feed-Back Analysis

- Degree of Performance

- Simulation Modeling

- Solution

Air and Hydro foil resources

http://airfoiltools.com/

UIUC Airfoil Data Site

Material Data