MATE ROV - OCEANUS

Description.

The goal of the simulation is to model the pressure  distribution caused by the incoming current, which has a  maximum force per area output of 1027N/m^2.

From the applied pressure distribution, stress and strain at  the hinges and walls have been measured.

Simulation of Chassis  Prototype Pressure  Distribution

Date: Sunday, February 22, 2026

Designer: Joseph Harb

Study name: Pressure Distribution

Analysis type: Static

Table of Contents

Description .......................................... 1 Assumptions......................................... 2 Model Information ................................. 3 Study Properties.................................... 4 Units ................................................. 5 Material Properties ................................ 6 Loads and Fixtures ................................. 7 Connector Definitions ............................. 7

Interaction Information ...Error! Bookmark not  defined.

Mesh information................................... 8 Sensor Details......................................11 Resultant Forces ..................................12 Beams...............................................12 Study Results ......................................13 Conclusion..........................................15

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 1

MATE ROV - OCEANUS

2/22/2026

Assumptions

Comments:

Assume that the current acts exactly perpendicular to the surface. This models an overestimation of the  stress, and the real effects will not be as exaggerated.

Although the real current varies and is time dependent, this static analysis models the maximum possibly  deformity resulting from the current itself.

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 2

MATE ROV - OCEANUS

2/22/2026

Model Information

Model name: Chassis Prototype (400 mm fixed)

Current Configuration: Default

Solid Bodies

Document Name and

Reference Treated As Volumetric Properties Document Path/Date  Modified

Force Applied to Bodies

Solid Body Fixture

Solid Body

Mass:3.852 kg

Volume:0.003852 m^3 Density:999.999 kg/m^3 Weight:37.7496 N

Mass:0.588369 kg

Volume:0.000588369 m^3 Density:1,000 kg/m^3 Weight:5.76602 N

C:\Users\josep\OneDrive - Texas A&M

University\MATE

ROV\Final Assembly (20  20 26)\Chassis Prototype  (400 mm fixed).SLDPRT Feb 22 00:08:46 2026

C:\Users\josep\OneDrive - Texas A&M

University\MATE

ROV\Final Assembly (20  20 26)\Chassis Prototype  (400 mm fixed).SLDPRT Feb 22 00:08:46 2026

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 3

MATE ROV - OCEANUS

2/22/2026

Study Properties

Study name Pressure Distribution Analysis type Static

Mesh type Solid Mesh Thermal Effect: On

Thermal option Include temperature loads Zero strain temperature 298 Kelvin

Include fluid pressure effects from SOLIDWORKS  Flow Simulation

Off

Solver type Automatic

Inplane Effect: Off

Soft Spring: Off

Inertial Relief: Off

Incompatible bonding options Automatic

Contact penalty stiffness scale factor 1

Large displacement Off

Compute free body forces On

Friction Off

Use Adaptive Method: Off

Result folder SOLIDWORKS document (C:\Users\josep\OneDrive - Texas A&M University\MATE ROV\Final Assembly

(20 20 26))

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 4

MATE ROV - OCEANUS

2/22/2026

Units

Unit system: SI (MKS) Length/Displacement mm Temperature Kelvin Angular velocity Rad/sec Pressure/Stress N/m^2

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 5

MATE ROV - OCEANUS

2/22/2026

Material Properties

Model Reference Properties Components

Name: 80% PETG

Model type: Linear Elastic Isotropic

SolidBody

1(Fillet113)(Chassis

Default failure  criterion:

Unknown

Prototype (400 mm fixed)), SolidBody 2(Cut

Extrude47)(Chassis Prototype

(400 mm fixed))

Curve Data:N/A

Comments:

PETG 80% Infill,

Yield strength: 2e+07 N/m^2 Tensile strength: 2.3e+07 N/m^2 Elastic modulus: 1.7e+09 N/m^2 Poisson's ratio: 0.35

Mass density: 1,000 kg/m^3

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 6

MATE ROV - OCEANUS

2/22/2026

Loads and Fixtures

Main Fixtures Fixture Image Fixture Details

Entities: 4 edge(s),

77 face(s)

Type: Fixed

Electronics

Enclosure Fixed Resultant Forces

Geometry

| Components | X | Y | Z | Resultant |
| --- | --- | --- | --- | --- |
| Reaction force(N) | -0.00400313 | 56.9111 | -0.018106 | 56.9111 |
| Reaction Moment(N.m) | 0 | 0 | 0 | 0 |

Side Walls Load Image Load Details

Entities: 2 face(s)

Type: Normal to

selected

Pressure on

Both Faces

Connector Definitions No Data

face

Value: 1,027

Units: N/m^2

Phase Angle: 0

Units: deg

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 7

MATE ROV - OCEANUS

2/22/2026

Mesh information

Mesh type Solid Mesh

Mesher Used: Blended curvature-based mesh Jacobian points for High quality mesh 16 Points

Maximum element size 30.4222 mm Minimum element size 1.52111 mm Mesh Quality High

Mesh information - Details

Total Nodes 160252

Total Elements 93827

Maximum Aspect Ratio 754.7

% of elements with Aspect Ratio < 3 96.8

Percentage of elements with Aspect Ratio > 10 0.439

Percentage of distorted elements 0

Time to complete mesh(hh;mm;ss): 00:00:08

Computer name: JOSEPHHARB

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 8

MATE ROV - OCEANUS

2/22/2026

Mesh Quality Plots

Comments: The mesh quality is rated high, corresponding to an accurate simulation. The highest aspect  ratios are located within the electronics enclosure, which has been modeled as a fixture, and does not  interfere with the results of the simulation itself.

Name Type Min Max Mesh Density Mesh - - Chassis Prototype (400 mm fixed)-Pressure Distribution-Quality-Quality1

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 9

MATE ROV - OCEANUS

2/22/2026

Name Type Min Max

Mesh Quality Aspect Ratio 1.025e+00 Element: 21244

7.547e+02

Element: 50223

Chassis Prototype (400 mm fixed)-Pressure Distribution-Quality-Quality2

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 10

MATE ROV - OCEANUS

2/22/2026

Sensor Details

Mesh Details:

Study name: Pressure Distribution (-Default-) Mesh type: Solid Mesh

Mesher Used: Blended curvature-based mesh Jacobian points for High quality mesh: 16 points Max Element Size: 30.4222 mm

Min Element Size: 1.52111 mm

Mesh quality: High

Total nodes: 160252

Total elements: 93827

Maximum Aspect Ratio : 754.7

Percentage of elements\with Aspect Ratio < 3 : 96.8 Percentage of elements\with Aspect Ratio > 10 : 0.439 Percentage of distorted elements : 0

Number of distorted elements:0

Reuse mesh for identical bodies: Off

Number of bodies that have reused mesh:0 Time to complete mesh(hh;mm;ss): 00:00:08 ---------------------------------

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 11

MATE ROV - OCEANUS

2/22/2026

Resultant Forces

Reaction forces

Selection set Units Sum X Sum Y Sum Z Resultant Entire Model N -0.00400313 56.9111 -0.018106 56.9111

Reaction Moments

Selection set Units Sum X Sum Y Sum Z Resultant Entire Model N.m 0 0 0 0

Free body forces

Selection set Units Sum X Sum Y Sum Z Resultant Entire Model N -0.241517 -0.0393057 0.0940476 0.262145

Free body moments

Selection set Units Sum X Sum Y Sum Z Resultant Entire Model N.m 0 0 0 1e-33

Beams

No Data

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 12

MATE ROV - OCEANUS

2/22/2026

Study Results

Name Type Min Max

Stress1 VON: von Mises Stress 0.000e+00N/m^2 Node: 119372

9.848e+05N/m^2 Node: 105141

Chassis Prototype (400 mm fixed)-Pressure Distribution-Stress-Stress1

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 13

MATE ROV - OCEANUS

2/22/2026

Name Type Min Max

Displacement1 URES: Resultant Displacement 0.000e+00mm Node: 10

5.891e-02mm Node: 116058

Chassis Prototype (400 mm fixed)-Pressure Distribution-Displacement-Displacement1

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 14

MATE ROV - OCEANUS

2/22/2026

Name Type Min Max

Strain1 ESTRN: Equivalent Strain 0.000e+00 Element: 70316

2.974e-04

Element: 15269

Chassis Prototype (400 mm fixed)-Pressure Distribution-Strain-Strain1

Conclusion

The deformation of the body is extremely exaggerated to emphasize effects.

The maximum displacement of the entire body was 0.05891 mm, which is negligible for our purposes.

The main thing to take away from this simulation is that the geometry connecting the electronics  enclosure to the chassis will not deform or break from external pressure to the walls of the chassis.

The thickness of the walls themselves are adequate and will not deform against the provided current.

Analyzed with SOLIDWORKS Simulation Simulation of Chassis Prototype (400 mm fixed) 15