- Subteams must present  their detailed design for each component.

  - SBC - Raspberry Pi 5

    - serves as the primary hub for everything, acts as the control center for the entire ROV, houses operating systems, integrates with sensors, etc.

    - Common system used for controlling, highly compatible with many systems

    - 2.4 GHz processor allowing it to run highly complex applications.

    - does have a slight lack of security, but having a secure password and regular security scans can prevent this.

    - also tends to overheat, but having cooling methods in place can prevent this.

  - IMU - BlueRobotics Navigator

    - Native support for BlueOS, very accurate 6 DOF IMU

    - Has built in IMU, compass, barometer

    - Might consider Pixhawk or blue robotics navigator for plug-and-play functionality with ardusub and blueos.

      - Adafruit option can still work but will require modifications to the firmware and is cheaper than the flight controllers

    - equipped to accurately describe the ROV’s real time position and speed accurately, being able to effectively control the ROV.

    - Also works with CNC, both teams agreed upon.

    - Includes a ST LSM6DS3TR-C, an entry-level 6-DoF IMU accelerometer + gyro (combines linear and rotational motion to determine position and velocity)

    - can have magnetic interference with other sensors but can be mitigated by keeping them relatively separate from each other

    - can have gyroscopic drift but typically doesn’t occur when have a strong sensor

  - Camera - BRIO 100

    - Might have to consider using the BRIO 4K due to new requirements regarding the photogrammetry

      - 4k feed is better for photogrammetry since it needs higher quality cameras.

    - Compatible with CNC, and matches needs for effectively having wide coverage vision across the ROV.

    - Has a fixed focus good for completing ROV tasks with multiple resolutions: 1080p/30fps (1920x1080 pixels) and 720p/30fps (1280x720 pixels).

    - Dual camera setup, 4k for photogrammetry, brio100 for streaming and navigation

  - Software - BlueOS

    - This is a custom Linux OS similar to Raspbian that has the necessary components for video streaming and taking inputs remotely for operating an ROV.

    - The interface is highly customizable and can take inputs from the control station and from the sensor suite.

    - The camera feed on the ROV can be sent live to the web server dashboard accessible on the control system.

    - Supports multi camera systems and can use BRIO 4k as photogrammetry camera and BRIO 100 for live feed.

  - Leak sensor

    - C&C is taking this

  - Depth sensor

    - C&C is taking this

  - Explain how it will be fabricated, tested and integrated into the overall system.

  - Provide a table showing which analysis and tests will be necessary to confirm each requirement is met.

- Subteams must present their chosen solution including specifications, final materials, and compatibility with other subsystems.

  - Subteams must clearly state why their design works and connect their choices to the requirements.

  - Subteams must show quantitative analysis as evidence that the solution will meet the requirements.

    - Ex: illustrations, modeling, simulations, calculations, analysis that support the chosen design.

- Subteams must address risk reduction.

  - Identify risks (technical, safety, etc).

  - Identify plans for risk reduction.

- Subteams should not go over 10 minutes