# Mission Overview

This document provides the overview of software development initiatives for the ROV. It documents a plan of action for each required task as well as testing and implementing created software.

Objective: Execute inspection, maintenance, and scientific tasks across three distinct operating environments (Flume Tank, Wave Tank, Ice Tank) using a distributed control system.

| GNC | C&C |
| --- | --- |
| Real-time control of the ROV, sensor fusion, safety failsafes, and hardware abstraction. | Ensure the reliability of communication/data transfer within the ROV. |

Communication: ROV connected to the control system via ethernet.

# Software Plans

Pre-mission setup:

| GNC | C&C |
| --- | --- |
| Hardware Calibration: Calibrate the IMU (check PID loops and do tuning in QGroundControl) Safety: Do not enable the ROV thrusters until C&C sends a command | Hardware calibration: calibrate the depth sensor to ensure it is giving accurate readings. Adjust the camera lights to give a clear image. Test input from the controller and ensure it is moving the ROV as intended. |

Mission Phase:

General objectives that must be completed and consistent through each required task.

| GNC | C&C |
| --- | --- |
| Motion Control: Receive joystick vector input from C&C, combine IMU and depth data to determine current state of ROV, and calculate PWM signals to thrusters. Current Compensation and Weight Pickup: Ensure that the ROV stays stable and has current hold and input hold for when picking items up and detect drift. | Camera feedback: Receive camera data, process through the microcontroller and send it through the tether to output onto the C3 station screen for the pilot. |

Task 1

Summary: Collect a set of species from a coral garden and create a 3D model of coral garden environment

| GNC | C&C |
| --- | --- |
| 1.1) Use navigational camera to provide line of sight for ROV to pick up two basket stars and species from bottom of pool 1.2) Implement photogrammetry with the designated camera system to collect visual data to configure a 3D model of the coral garden.Use IMU to determine dimensions of coral garden 1.3) Maintain the ROV at a stable position to accurately obtain video | 1.2) Receive designated task camera feed and format the data into multiple different still images. (Or screenshot, will discuss later) 1.3) Implement a “record” system into the C3 station to allow for live feed to be recorded and saved. |

Task 2

Summary: Determine number of invasive species and profile iceberg.

| GNC | C&C |
| --- | --- |
| 2.1) Ensure the ROV stays still to allow capture of a clear image at five different points of the iceberg. General positioning part of general mission phase objectives 2.2) Readout high-resolution depth data for the keel-profiling for iceberg. 2.3) Measure keel depth, location, and heading utilizing IMU like done in Task 1 to determine threat to each oil platform | 2.1) Process designated task camera feed on C3 station and implement a form of image recognition software to display a bounding box for each green crab along with the number on display. 2.2) General Objective: Camera feedback for the five points. Receive depth sensor data, process through the microcontroller, send through tether into C3 station, and output it. |

Float

Separate from ROV and autonomously send data back to mission station

| GNC | C&C |
| --- | --- |
| 4.1) Send test data packet before vertical profile 1 via bluetooth to control system 4.2) Collect data during vertical profile and save to SD Card 4.3) Transmit saved data to control system via bluetooth | 4.1) Receive data packet pre-mission, format it, and output it on C3 Station. Then receive all of the recorded data packets post-mission and do the same. Have the microcontroller receive the depth readings and tell the buoyancy engine what to do based on the depth readings. Have the ultrasonic sensor send the distance from ice to the microcontroller to also factor into what the buoyancy engine should do. Make float automatically start transferring data packets when it emerges. |

# Testing

GNC

| Category | Testing Criteria |
| --- | --- |
| Motion | 360 Test - 360 Test with cameras attached to the ROV - check that connections are not coming loose for camera while rotation |
| Vision | Camera - test for image quality - test for stability on camera in movement - check for shifting of camera while rotation Precision Test - Check cameras to see if images come blurred or not - Test how precise movements are to specific locations |
| Control | Speed/Control - test various speeds such that it can maintain clear images, intact parts, etc. |

C&C

| Category | Testing Criteria |
| --- | --- |
| Sensors | Depth Sensor - Test accuracy in cold and regular temperatures Ultrasonic Sensor - Test accuracy in cold temperatures |
| Vision | Camera - Test the photo recognition for the _ camera - Test camera output for both cameras in relation to lights Lights - Test to make sure the lights are a good brightness for clear camera quality |
| C3 Station | Live Feed - Test for navigational camera feed properly displaying - Test the photo recognition output on the display Controller - Test button commands |
| Communication | Bluetooth - Check reliability of data transfer Microcontroller - Test to make sure the Pi-5 is able to handle all the data we will be sending through it (4k images, 1080p camera feed, etc.) |