https://www.foamcoreprint.com/blog/best-stereo-camera-for-depth-perception-in-imaging-and-robotics?srsltid=AfmBOop9FHDRIcrBXq3z1l3fD0QfeD9QD_Sg5oqHFEFq2tnpvosmnOvt

Stereo cameras offer depth sensing to analyze disparity between two images to analyze their depth

Approaches to perceiving depth

- passive stereo:

- active stereo

- structures light

- time of flight

Stereo cameras tend to be good with accuracy and with tracking moving objects. has a high frame rate, but can be poor in low light conditions.

Time of flight tends to have better low light conditions

stereo cameras more cost-effective but due to their low-light conditions can often be paired with other light methods to increase visibility

Camera Systems

Fixed Baseline Stereo:

- Two cameras side by side, horizontally oriented

- Depth is calibrated from position

- Limited - only get straight forward visibility

Auxiliary Stereo

https://reachrobotics.com/blog/what-is-an-rov-camera/

- One camera fixed for general visibility while another one added for more specific ROV tasks, have a main ROV camera and another camera on another viewpoint

- Good optics/view

- good usability

Convergent Stereo

- Two cameras pointed inward

- depth perception can be controlled

- can be used to create a 3D effect

- depth sensing

- used a lot in autonomous vehicles

- can be difficult to use

https://voyis.com/photogrammetry-stereo-vs-single-camera/

Stereo cameras used more often with ROVs

Can create accurate real time depth perception, and are easier to calibrate and use as opposed to more monocular camera systems

Camera Housing

https://www.homebuiltrovs.com/seafoxcamera.html

Plexiglass covering on top with enclosing around camera

Needs to be watertight

Glass super close to lens so dont mess with quality

Lens

- Use a dome part

- better to not

- https://photographylife.com/underwater-housing-camera