Task 2 Points Breakdown

Task 2.1 Produce Power

- Connect a floating solar panel array to the grid

  - Retrieve the power connector – 5 points

  - Install the power connector into the hub

    - Remove the cover from the connection port – 5 points

    - Install the connector – 10 points

- Protect an offshore wind farm from corrosion

  - Replace a sacrificial anode onto the base of an offshore wind farm

    - Disconnect the old sacrificial anode from the base – 10 points

    - Connect the new sacrificial anode onto the base – 15 points

  - Mitigate corrosion

    - Conduct a visual inspection of the base structure – 5 points

    - Apply an underwater epoxy patch to the corroded area – 10 points

Task 2.2 Monitor Environmental Impact

- Collect life stages of jellyfish

  - Collect polyp stage jellies attached to the solar panel array

    - Collect 4 or more polyps – 10 points

    - Collect 1 to 3 polyps – 5 points

  - Collect a medusa stage jelly from mid-water

    - Visually determine medusa jelly is in open water – 5 points

    - Collect the medusa jelly – 15 points

- Collect fish species aggregated underneath the solar panel array

    - Collect 4 or more fish species – 10 points

    - Collect 1 to 3 fish species – 5 points

- Place a hydrophone in the designated location

  - Remove the hydrophone from the platform – 5 points

  - Place the hydrophone in the designated location – 10 points

  - Pull the pin to release the hydrophone from its base – 5 points

Total points = 120 points

Task 2.1 Produce Power

Objective: Connect a floating solar panel to the grid and protect an offshore wind turbine from corrosion.

- Retrieve the Power Connector

  - Objective: Remove power connector (1" PVC) from underwater platform.

  - Important:

    - Use ROV to take full control (not touching platform or bottom).

    - Connector is attached via a #6 hook and wire.

    - No part of it can touch the bottom/platform after retrieval.

- Remove Connection Port Cover

  - Objective: Remove 3" PVC cap from 2" PVC port.

  - Important:

    - Can be done before or after retrieving the connector.

    - Must be fully under ROV control and no longer touching ports.

    - Treated as debris → must either stay with ROV or be returned to the surface.

- Install the Power Connector

  - Objective: Insert Velcro-covered connector into Velcro-lined port.

  - Important:

    - Velcro hooks are on the connector; Velcro loops are inside the port.

      - You need a tight, aligned fit to ensure Velcro engagement.

    - Must be:

      - Not touching ROV.

      - Fully inserted.

      - Velcro-to-Velcro secure stick.

    - Must remain installed the entire run or you lose points.

    - You can reinstall if it becomes dislodged.

- Replace Sacrificial Anode

  - Objective: Swap old silver-painted PVC anode with a new one.

  - Important:

    - Both anodes are painted silver, so color won’t help distinguish.

    - Removal:

      - Rotate until PVC rods align with slot, then remove.

        - The disconnect method is mechanical: Turn/rotate the old anode until its ½-inch rods align with a slot in the corrugated plastic.

      - Must be under ROV control and not touch base afterward.

      - Considered debris – return to surface or keep with ROV.

    - Installation:

      - Insert both ½" PVC sections into slot below plastic sheeting.

        - On insertion of the new anode: The PVC rods must go fully through the slot, and be below the sheeting.

      - Must remain in place for full run or lose points.

      - You can reinstall if it falls out.

    - Importance of alignment and full insertion:

      - Failure to align the rods properly may result in the anode falling out, costing you points.

- Mitigate Corrosion by Applying Epoxy Patch

  - Objective: Find and patch corrosion on one “leg” of wind base.

  - Important:

    - Inspection:

      - Find orange paint (≥1.5 cm wide) on one leg (not underside).

      - Show corrosion on video feed to judge.

        - This must happen before applying the patch.

    - Patch:

      - The patch is a cloth towel with Velcro loops on each end.

      - Dish towel with Velcro loops must:

        - Attach above and below spot.

          - The “legs” have Velcro hooks, and both ends of the towel must attach.

        - Fully cover the corrosion.

      - Must stay on for the entire run.

Task 2.2 Monitor Environmental Impact

- Collect Polyp-Stage Jellies

  - Objective: Remove black chenille polyps from holes in solar array PVC and return to surface.

  - Important:

    - Polyps are:

      - Made from black chenille pipe cleaners

      - Inserted into 3/16-inch holes in the solar array’s PVC frame

      - Small and likely delicate: Ensure your manipulator doesn’t bend or break them during extraction.

    - Must be under ROV control and fully removed from panel.

    - Must be placed at surface, side of the pool.

- Collect Medusa-Stage Jelly

  - Objective: Release jelly (water wiggler), observe it float, then collect it.

  - Important:

    - Pull pin to release from 3" PVC holder.

      - Enclosed in a 3-inch pipe holder at the start.

      - Must pull a ½-inch pin to release the jelly.

    - Must visually confirm jelly is floating (not touching holder/framework).

    - Show floating state to judge on video.

      - DO NOT COLLECT UNTIL THIS STEP IS DONE!

    - No direct pressure on jelly body during collection.

      - Collect surrounding water with jelly.

    - Return entire collection mechanism (with jelly inside) to surface.

      - You can surface the ROV with the jelly inside or detach the container and send that up.

    - Never remove jelly from container during run.

- Collect Fish Species

  - Objective: Remove ping pong balls from PVC frame under solar array.

  - Important:

    - Fish are floating within a square of ½-inch PVC.

    - You need to remove and bring them to the surface, side of the pool.

      - Suggest using either a scoop net, cage, or suction system.

    - Ensure you don't just dislodge them:

      - Must be under ROV control and returned to surface.

    - Ensure complete removal from the floating structure.

- Place Hydrophone

  - Objective: Move hydrophone from platform to yellow square and release sensor.

  - Important:

    - The hydrophone:

      - Starts on a platform on the bottom

      - Made from ½-inch, 2-inch, and 3-inch PVC

    - Remove from platform:

      - No longer touching platform or pool bottom.

    - Place into yellow square:

      - Must be upright and completely within yellow 50x50 cm square.

      - No part of base can touch or be over PVC edges.

      - Must remain upright and in bounds throughout the run.

      - Can reposition if it gets knocked/moved.

    - Release:

      - Pull pin so 2" sensor floats above 3" base attached by rope.

      - Show sensor floating to judge on video.

      - Even if hydrophone was knocked over earlier, you can still get these 5 points.

Critical Guidelines to Avoid Losing Points

- Debris: Port cover and old anode must be with ROV or returned to surface at end.

- Permanent Placement:

  - Power connector, anode, patch, and hydrophone must stay in place/upright until end.

  - Points are lost if items are dislodged.

- Video Verification:

  - Corrosion ID and medusa jelly must be shown to station judge via ROV camera.

- Medusa Handling:

  - Do not squeeze, press, or directly grab jelly body.

- Accuracy on Hydrophone:

  - Must be upright and fully inside the yellow square. Can be repositioned if needed.

General Operational Insights

- During the 5-minute setup period, some task components like:

  - The new sacrificial anode

  - The epoxy patch

  - And potentially collection containers

will be located at the surface, side of the pool.

    - You must collect and transport them using your ROV at the beginning of the mission.

- All debris such as:

  - The port cover

  - The old anode

must either be returned to the surface or be under control of the ROV at the end of the run.

    - Loose debris left at the bottom will likely result in a point deduction.

- If a task item becomes dislodged (e.g., power connector, patch, anode, hydrophone), you can attempt to reinstall it to regain the lost points.