Research Notes - MATE ROV

Date:

06/06/2025

Researcher(s):

Bhavana Shree Pothuri

Objective of research:

Notes on the previous teams EPS report.

Notes:

## Electrical System – Key Concept

- Goal: Flexibility – modular, understandable, reliable.

- Enables future teams to easily upgrade/change tools.

- Modular design supports easier testing and debugging.

## System Overview

- Two decentralized subsystems:

  - Topside Control Station

  - ROV-Underwater-Box

- Communication via MQTT over Ethernet.

- Subsystems work independently but integrate as one system.

- Modularity enables:

  - Easy testing of individual modules.

  - Easy addition/removal of features.

## Topside Control Station

- Laptop runs GUI and displays camera feed.

- Connected peripherals:

  - Monitor (HDMI)

  - Xbox Controller (USB) for ROV control

## ROV-Underwater-Box

- Single waterproof aluminum box (functions as a heatsink).

- Housing issues:

  - Cast part had draft angles → professionally milled.

  - Epoxy resin added for sealing due to threading issues.

- Core electronics:

  - Self-designed PCBs connected via a backplane for easy mounting and wiring.

  - STM32H743 NUCLEO – handles PWM + sensor signals.

  - Raspberry Pi – communication with STM (via UART).

- Modular boards allow:

  - Keeping base shields.

  - Replacing only tool/sensor-specific boards.

## Custom PCB Modules

### Power Distribution Unit (PDU)

### Converts 48V from tether to:

### 5V/6A (now 12A)

### 3.3V/1A

### Powers sensors, microcontrollers, servos.

### Motor Control Unit (MCU)

  - Custom buck converters → 22V output.

  - Each motor shield: 12A max, 7kg thrust.

  - Main board handles current measurement, fuse detection, LED indicators.

  - Modular design for quick shield replacement.

- Sensor Control Unit (SCU)

  - Purpose: Signal processing (not direct control).

  - Key components:

    - MAX11210 (24-bit ADC)

    - LM4040 (2.5V reference)

    - ASM330LHH (6-axis IMU)

  - Supports:

    - 4-20 mA current sensors (e.g., pressure sensors)

    - Resistance sensors (e.g., PT100 temp sensor via Wheatstone bridge)

    - IMU for control optimization (SPI/I2C communication)

- Tool Control Unit (TCU)

  - Supplies power/PWM/signal to tools (e.g., grippers).

  - Offers GPIO, SPI, I2C, etc., for future expandability.

## Electronic Issues (Version 1)

## MCU: Overheating, chip failure at full power.

- Misrouting on PCBs → extra soldering.

- PDU: Insufficient power delivery, especially for Raspberry Pi.

- Wiring needed improvements.

## Underwater Box 2 – Improvements

- Centralized hardware, clearer wiring.

- Features:

  - No splices, structured wiring.

  - 5V power doubled (12A → split: 6A critical, 6A mission).

  - Shunt & Hall-effect current sensors (servos/thrusters).

  - Extra fuses for mission-specific servos.

  - Direct UART to Raspberry Pi.

  - Debug headers for communication.

  - More signal LEDs for diagnostics.

## Tether Design & Management

- Goals: Minimize drag, weight, entanglement, voltage drop.

- Design:

  - 15m long

  - 2x AWG10 power wires

  - 1x Ethernet cable

  - Spiral wrap + strain relief + cable guide

- Strain relief: Custom-designed to secure cables.

- Cable guide: PVC and glass bearings, self-made.

- Buoyancy: Pipe insulation added to keep tether on the surface.

- Routing: Kept close to ROV frame to avoid snags.

General Essential Considerations for Building Underwater Rover:

- Modular & Flexible Electrical Design

  - Design systems to be modular, reliable, and easy to understand.

  - Enables future upgrades and rapid integration of new tools or sensors.

  - Independent, self-contained subsystems (e.g., topside station, ROV box) improve testability and scalability.

- Robust Communication System

  - Use Ethernet and MQTT for real-time, reliable communication between surface and underwater components.

  - Ensures flexibility without needing to change physical tether wiring.

- Topside Control Simplicity

  - Use a laptop with GUI, monitor, and game controller (e.g., Xbox) for user-friendly control.

  - Centralized control simplifies operation and software integration.

- Waterproof & Thermally Managed Electronics Housing

  - Use a single waterproof housing (e.g., aluminum) to protect electronics and act as a heatsink.

  - Account for manufacturing flaws (e.g., casting angles) and use epoxy or custom machining for reliable sealing.

- Power Distribution and Conversion

  - Convert high-voltage tether input (e.g., 48V) to lower voltages (e.g., 5V, 3.3V) for microcontrollers and sensors.

  - Include fuses, current sensors, and LED indicators for protection and diagnostics.

- Motor & Sensor Control

  - Use custom Motor Control Units (MCUs) for efficient power conversion and thruster control.

  - Employ high-resolution ADCs and Sensor Control Units (SCUs) to read various sensor types (current loops, resistance-based, IMUs).

  - Add Tool Control Units (TCUs) for actuators like grippers or lights.

- Testing and Iteration

  - Expect and plan for hardware revisions.

  - Identify and fix issues such as overheating, misrouting, insufficient power, and wiring complications.

  - Debug headers and signal LEDs greatly aid in troubleshooting.

- Efficient Tether Design

  - Design tether to minimize drag, entanglement, and voltage drop.

  - Use proper gauge wires (e.g., AWG10), spiral wrapping, and strain relief.

  - Add pipe insulation for buoyancy and custom cable guides to avoid interference with ROV movement.

Sources:

University of Applied Sciences Esslingen Technical Documentation.