Research Notes - Mate ROV

Date: 06/06/2025

Researcher(s):

Khalid Abou-Khamis

Objective of research:

Notes on the previous teams EPS report.

Notes:

### Electrical Design Overview

- The team used SID (System Integration Diagrams) to map power and signal connections.

### Power Supply

- Input: 48V DC power supply from topside, converted to 12V onboard for components.

- Used an off-the-shelf OmniOn OBDS128A0B: Barracuda DC-DC Converter (1.5 kW capacity) due to space, time, and efficiency constraints.

  - Small ¼-brick form factor

  - High power density

  - Never overheated in testing

### PCB and Connectors

- Custom PCB designed using KiCad for tight spatial constraints and high current routing.

- Blue Robotics Wetlink Penetrators used for waterproof cable entry.

## Components and Tools (Electrical)

### Thrusters

- 6x Blue Robotics T200 Thrusters

  - Brushless motors: 12V, 20.4A peak, ~35 N thrust

  - Durable, efficient, and bidirectional

- Controlled via ESCs (Electronic Speed Controllers)

  - Quick connectors and protected signal cables were added for modularity.

## Electrical Safety Practices

### General Electrical Safety

- Low and high-voltage components are kept separate and powered off when not in use.

- Strict guidelines followed to prevent injuries and component damage:

  - Circuit breaker (40A) + fuse (30A) setup

  - Power is disconnected and breaker tripped during electrical work

- Power wiring includes strain reliefs to prevent dangerous cable tension or detachment.

### Topside Safety Features

- Custom pelican case houses electronics

- Organized into top (frequent-access) and bottom (less-accessed) sections with steel barriers

- HDMI, pneumatic, and tether connectors clearly labeled and managed

### Safety Checklist (Appendix A3)

Includes pre-launch, launch, and post-launch protocols:

- Secure penetrators, tether strain relief, and check all electrical connections

- Confirm no bare wires or loose screws

- Emergency shutdown procedures in place

- Components must be dried and protected post-launch

## Troubleshooting & Lessons Learned

- Buck converter overheated under heavy load → contact with aluminum tray improved heat dissipation.

- Ultimately found power draw exceeded converter limits → software limited acceleration to avoid shutdowns.

- Jetson occasionally sent incorrect PWM signals → reverted to Navigator + Pi due to timing stability and support.

Sources:

University of Texas at Dallas Technical Documentation.