Research Notes - Mate ROV

Date: 06/14/2025

Researcher(s):

Khalid Abou-Khamis

Objective of research:

Safety Requirements of the Rover

Notes:

General Wiring and Connection Quality

● All circuit boards are securely mounted, not dangling.

● All electrical connections are made by soldering or terminal block with screw or spring locks (pin headers are prohibited for final delivery).

● All exterior copper or exposed wires are properly insulated.

● Wire routing maintains separation between high power (motor, battery) and low power (control signals).

● Whenever high power crosses low power lines, they do so at right angles to reduce interference.

● All wires are secured with strain relief or rubber grommets when passing through an enclosure wall.

● Encapsulating boards in resin or resin-like compounds is fine, but they must be secured, not left dangling.

Batteries and Power Supply

● All batteries are safely secured in a strong, non-flammable battery pack (metal or hard resin enclosure).

● Batteries are not taped together or left unsupported.

● Fuses or circuit breakers are rated appropriately (typically 150% of max current but not greater than 30 amps) and placed close to the main power connection.

● All main power switches, fuses, and protective elements are easily accessible for emergency shutdown.

Motor Sealing (Brushless) and Waterproofing

● All brushless motor connection points are fully encapsulated in resin or resin-like material to avoid water seepage.

● Encapsulation resin is electrically insulating and thermally conductive.

● A pressure-test (40 psi or less) after curing should show >100 MΩ resistance between motor leads and water, demonstrating proper sealing.

● All motor leads are routed safely and secured.

Safety Procedures and Inspection

● All team members are trained and aware of lockout/tagout (LOTO) procedures.

● All team members follow proper strain relief and routing practices to avoid mechanical stress.

● All high-current lines and components are kept clear from sharp edges or potential abrasives.

● All components are visually inspected for damage (cuts, abrasions, frayed edges) before each competition or water entry.

Sources:

HSE Handbook - 4_19_2018

MATE ROV Competition Tech Bulletin - MATE Expected Work Practices MATE_ROV_Competition_Tech_Bulletin_Sealing_Brushless_Motors https://materovcompetition.org/safety