BlueOS Deployment Notes

Date:

7/12/2025

Researcher(s):

Arun Vaithianathan

Objective of research:

Testing BlueOS on available hardware.

Notes:

System Setup

- BlueOS successfully installed on Raspberry Pi 3B.

- Setup is simple and cross-platform (Windows/macOS).

- Raspberry Pi Camera connected; OpenCV testing planned.

Control Station & Connectivity

- Dedicated Windows PC will act as control station.

- Tethered via Ethernet to ROV; provides internet access for BlueOS updates.

- BlueOS hosts a local web server for full ROV control.

Web Interface Features

- Integrated video streaming and controller setup.

- Supports autopilot firmware flashing (e.g., for IMUs, flight controllers).

Performance

- Pi 3B’s 100 Mbps Ethernet is fully utilized during streaming — sufficient for video.

- Software appears well-optimized for bandwidth limits.

Extensions

- Built-in “Extensions” tab acts as app store.

- ROS extension available for advanced robotics integration.

Hardware Constraints

- Pi 3B (1 GB RAM) struggles with video + extensions.

- Video feed occasionally fails to load.

- Recommend Pi 4B or 5 with 4–8 GB RAM for stable performance.