Cameras - GNC

6/19/25:

Researcher(s):

Mihika Varshney

Objective of research:

Research into usage of cameras for the ROV for better navigation

Past Team Reports

Triton Tech Robotics

- Consider a dual camera system. One for general movement while the other is for better visibility and to create more precise movements

  - Camera measurements: 61 degrees horizontal 47 degree vertical positioning

  - One faced forward, one faced towards manipulator

- Waterproofed by coating in epoxy resin and encased in acrylic cylinders

- Receptacle connectors control the camera

Camera layout

Esslingen University

- Usb camera modules: easy and cheap and can connect easily to a raspberry pi

- Epoxy resin used to waterproof camera

- 3 camera system to get an all around view of the ROV. up to 160 degree perspective

The University of Texas at Dallas Team Report

- 3-camera system: see front, underside, and claw and gripper

- Blue Robotics Low Light Camera: compact size makes them ideal

- Placed inside the main enclosure meaning not necessary to waterproof the cameras

- Created an external camera enclosure for other cameras

  - Had issues with enclosure bending due to pressure, changed from long edge feature type to radial curve with solved issue

Links

- Buy a Raspberry Pi High Quality Camera – Raspberry Pi

Sources:

Triton Tech Robotics Team Report

Esslingen University Team Report

The University of Texas at Dallas Team Report