## ROV Software & Hardware Architecture Considerations

Date:

7/11/2025

Researcher(s):

Arun Vaithianathan

### Shift Away from LabVIEW

- LabVIEW is primarily tied to the Barracuda Kit and Arduino-based control, limiting customizability.

- Python is preferred:

  - Faster development.

  - Easier integration with libraries like OpenCV and TensorFlow.

  - Natively supported in BlueOS and ROS2.

  - More team members are familiar with Python (e.g., from ENGR 102), enhancing collaboration and troubleshooting.

## Software Stack Recommendation

### BlueOS Overview

- Custom Linux-based OS optimized for ROVs and compatible with Raspberry Pi.

- Runs alongside ArduSub firmware on a flight controller/autopilot board.

- Built-in capabilities:

  - Video streaming

  - Sensor data integration (IMU, magnetometer, etc.)

  - Peripheral interfacing

- Open source and actively maintained:

  - GitHub repo: BlueOS

  - Documentation: BlueOS Docs

## Autopilot / Flight Controller Hardware

### Navigator Flight Controller (Blue Robotics)

- Designed for Raspberry Pi 4 (GPIO alignment specific).

- Integrates seamlessly with BlueOS.

- Key features:

  - Built-in 6-DOF IMU

  - Leak sensor support (potential tie-in to safety system)

  - Compatible with ArduSub firmware

- Commonly used in MATE ROV community.

- Recommended for:

  - Early purchase (limited stock, custom-manufactured)

  - Backup unit (in case of water damage)

Reference: Navigator Product Page

## Thruster & Motor Control

### T200 Thruster (Blue Robotics)

- Widely adopted underwater thruster.

- Best paired with brushless ESCs (e.g., ROVMAKER 80A ESC)

- 4-thruster setup is standard for good maneuverability and precision.

Thruster Link: T200 Thruster

## Communication & Tethering

### Fathom-X Tether Interface

- High-speed Ethernet-based tether communication.

- Sends video + sensor data to surface PC.

- Connects directly to the Raspberry Pi, not to the Pi’s USB port.

Tether Link: Fathom-X Tether Interface

## Sensor Data & Processing

- Typical sensor polling rate: 10–30 Hz

- OpenCV:

  - Converts camera input into NumPy matrices, making real-time processing easy.

  - Raspberry Pi 4 has sufficient performance for typical frame processing.

  - If overloaded:

    - Consider offloading frames via tether to the surface computer (introduces lag).

    - Frame buffer and RAM may become bottlenecks.

## Additional Considerations

- ArduSub-compatible boards can be custom flashed with the firmware.

- Navigator board provides easier and more reliable integration with BlueOS than generic flight controllers.

- Sonar Compatibility: Ping360 sonar is frequently used with Navigator boards.