SBC Research

Date:

7/7/2025

Researcher(s):

Arun Vaithianathan

Objective of research:

Find the correct SBC needed for our use case and ensure it is within a reasonable budget.

Notes:

Arduino Uno Rev3: [Link] ($27.60)

- This is the SBC included in the Barracuda ROV

- Extremely limited

- Does not have an ethernet port, only one USB type-B port

- I personally don’t think this is a good option, the I/O is very limited since we are already considering a multi camera set up which will require a more powerful CPU and bandwidth on the USB port (this is only USB 2.0)

Raspberry Pi 5: [Link] ($120)

- Much more performance on this board

- This has way more GPIO pins and even has an M.2 port

- Has ethernet ports and USB 3

- I personally would be ok to spend the extra money on this since it will give us head room in the future to run computer vision tasks if needed

Raspberry Pi 4B: [Link] ($60)

- Very good priced alternative to the Pi5 and has better compatibility with BlueOS

Raspberry Pi 3B: ($Free)

- This can be good to start immediate research and testing with CV, programming, etc.

- I am fine with using it for the team since I just haven’t been using it for anything else

- This has a similar feature set to the Raspberry Pi 5, just a weaker CPU and no M.2 port

- We may be unable to run CV on this SBC, but an TPU card such as the Google Coral can be purchased to run CV ($60)

NVIDIA Jetson Nano: ($200-$400)

- Like with all NVIDIA GPUs, there’s a stock shortage but the Jetson is very powerful

- It has a similar I/O set up to the Raspberry Pi but includes the GPU onboard which can enable complex CV tasks

  - This could be necessary if we aren’t using any sonar and rely only on CV

- The setup of this costs the same as a Raspberry Pi + a Google Coral

Sources:

https://coral.ai/products/accelerator