Think Tank MATE ROV - TMS

Date:

6/1/2025 - 6/3/2025

Researcher:

Nikhil Modak

Objective of research:

Researching previous competition teams (2023) to determine information on materials and mechanical subsystems

Notes:

* All teams used a strain relief system for the tether

Allen Hancock College

Materials / Structure:

- Frame: PVC Pipe, PVC Joints

- Motor Shrouds:  3D printed PLA (Strong and lightweight)

- Electronic Waterproofing: heat-shrink tubing, saran wrap, and

electrical tape

- Air Tanks and Clay weights used to control buoyancy

Issues:

- Rover sank straight to the bottom without buoyancy control like weights and air tanks

- Original motor shrouds didn’t hold up well so the team switched to PLA

Cabrillo College

Materials / Structure:

- Frame: 20x20 t-slot aluminum extrusions joined with custom-made  anodized aluminum brackets, all manufactured aluminum parts were made from grade 6061 aluminum and anodized to prevent galvanic corrosion when in contact with stainless steel mounting hardware,

PVC - strong, cheap, lacked design flexibility, affected buoyancy

Carbon Fiber - very strong, expensive, lacked design flexibility

- Ballast Tanks used for efficient buoyancy control

- Shroud guards: 3D printed Tether: Braided nylon sheath, Nylon webbing was braided around the tether and wrapped with silicone tape for durable strain relief

- Safety: 3D printed caps for sharp edges and corners of rover, set failsafes and safety check abilities in case rover needed to be shut down in emergency situations

Issues:

- Should have created a box for electronics for ease of manufacturing and minimized weight

Estrella Mountain Community College

Materials / Structure

- Frame: PVC pipe, makes prototyping easy, deforms when exposed to heat so it is easy to manipulate

- Robotic Arm: cylindrical stroke, rotary lever hand valve, hosing, and a 3D-printed claw, inspired by crab claw

- Electronic Waterproofing: sealed in epoxy

- Thruster Shroud Guards: 3D printed (make sure to maximize flow while still offering protection)

Issues:

- Took multiple iterations to perfect shroud guards as early iterations didn’t allow for enough flow in an out of the propeller

- Wires of different lengths caused complications when feeding wiring through the tether

Pasadena Community College

Materials / Structure

- Frame: High Density Polyethylene, corrosion resistance, easy maintenance, simple detachment of all components, HDPE exhibits a density comparable to water making it neutrally buoyant,

- Electronics Housing: Acrylic, Sealed using O rings, vacuum sealed to prevent leaks

- Thruster guard: 3D printed

- Static ballast system consisting of PVC pipes filled with sand

Issues:

- Thruster guard holes were too big and allowed debris and material to flow in, causing props to stall

Southwest Virginia Community College

Materials / Structure

- Frame: PVC pipe (modular style makes it easy to change design) and 3D printed PLA mounts

- Utilized a ballast tank to control buoyancy - PVC pipe

- Thruster guards were 3D printed

- Pneumatic gripper system used as a mechanical arm

- Utilizes GO-BGC Float (not really sure what this does but it was listed as part of the payload)

- Consider making all mechanical systems modular so things can be removed and attached with ease

Issues

- Servos for gripper system were very hard to waterproof, so the team switched to pneumatics (“Waterproof” servos are not necessarily waterproof)

- Determining the size of the ballast tank to achieve neutral buoyancy proved to be difficult

Universidad de los Andes

Materials / Structure

- Frame: PVC, rods and caps were aluminum

- Electronics Waterproofed tank - translucent acrylic sealed with O rings and capped with an aluminum cap

- Utilized vertical float to control buoyancy

- 3D printed, four finger claw is used to accomplish mission tasks

Issus

- 3D printed O rings proved to be less reliable than purchased ones

Sources:

https://materovcompetition.org/2023-competition-archive