Pioneer 2022 - TMS

Date: 6/5/25

Researcher(s): Alex Huynh

List the names of the team members contributing to the research.

Objective of research: Decision Making/Correlation with Scoring

Define the problem or research question you aim to address.

Notes

Cabrillo:

- Safety Equipment(Person)

  - Eye protection, face masks, eyewash stations, shower stations, first aid kits, and fire extinguishers.

- Safety(ROV)

  - Tether System - Master Fuse and Strain Relief Webbing

  - Edges covered by caps

  - Blue Robotics T200 Thrusters(4x) covered

  - Gripper(2x) has gear that removes objects above mechanical limit

    - ABS Filament

- Overview

  - Testing Electrical Housing through ANSYS then pool

  - 9.2 kg/Depth 15m/Aluminum & 3D Printing

  - Frame Goals: Modularity, Budget, Neutral Buoyancy, Ease of Use

  - Materials - Aluminum (Stiff Strong Cheap), excludes Carbon Fiber (Inflexible, affects buoyancy and expensive), PVC(Affects Buoyancy and Inflexible)

    - 3D Printing, then Machining, last resort CNC

  - Parts Placed to keep center of buoyancy in line of XY plane and above COM

  - Lifting handles(Human Convenience)/Skids(Friction Reduction)

  - Tether system that has strain relief and detachable

  - 1 Single Gearbox to house electronics

  - Used surfboard foam(secondary) and stainless steel floats for buoyancy

  - Ballast System for swift buoyancy adjustments

  - Pin Extractor(?) - unsure why

  - Buoyancy Engine using syringe plunger and servo system, and releases based on pressure

SW Virginia

- Frame - 3D Printing(PLA)/PVC Pipes(Light, inexpensive, durable, and replaceable)

- Tether - 50 ft, ethernet, airhose for buoyancy, rope for strain relief, and pool noodles to reach neutral buoyancy

- Thrusters(4x) - TriggerFish Four motor propeller kit

  - 2 Parallel

  - 2 mounted 45 degree angle

  - Ctrl-ed through Barracuda Ctrl Box Kit

- Buoyancy - Reached through flotation devices, such as PVC pipes, inner tubes ,and air hose

- Arms - made to bolt onto body, high infill for rigidity, parallel to each other, and tooth to prevent pins and ropes to slip out

  - Ctrl-ed through PS3 controller and ESPP32 board

Savanna Tech

- Frame - PVC/3D Printing

  - Octagonal w 3 rows of PVC for stability & attachments

  - Holes for Neutral Buoyancy

  - Cheap for Budget

- Thruster(6x) - covered, 2 thrust motors(Left/Right), 3 used for vertical

- Barracuda Kit modified

- Tether - 1 power & ethernet cable and 4 pneumatic hoses, and strain relief system

- Ballast System using Weights(Static)

- Vertical and Horizontal Grabber

- Pneumatics to stand to water pressure and no electrical shortage, run to grabbers

- Syringe connected to a balloon for buoyancy engine using latches and levers, inside a PVC, and timer

- Problems - Didn’t Sink(Used Static Ballast System) and Grippers not strong enough

- Safety - PPE and rounded edges

- Checklist for proper safety procedure

Pasadena:

- Frame - TriggerFish ROV Frame and PVC pipe

  - Trapezoidal and 45 degree vector design

- System Specifications

  - System specifications include:

  - - Must be able to fit in a 1 meter by 1 meter hole in the “ice.”

  - - Must weigh less than 35 kg (or 77 pounds).

  - - Vehicle must overcome 3 obstacles such as placing a fish into a bucket, replacing seagrass, and unhook the ghost nest.

  - - Materials compatibility requirements must be met to be used underwater.

  - - Do not exert a lot of force onto the ROV as there are limitations on the material.

- Thrusters(4x) - 2 motors (fwd/reverse) and yaw, 2 motors for Sway and Heave

- Arm Scooper, ABS Filament

- Safety - Waterproof, safety guards

- Tether - Strain Relief

Sources: Pioneer 2022 in MateROV folder