Supplier - McMaster

- Raw material and hardware mechanisms. Prices on raw materials are not the best rate, but they have incredibly fast shipping.

- Fasteners

Supplier - Mouser

- Everything we need for electrical components, like capacitors, etc.

Heat Transfer Tips

- Conduction (most efficient)

  - Through physical contact.

  - For whatever is generating a lot of thermal energy on the inside, give it a low thermal resistance to the outer walls, so that water outside can cool it. Put thermal paste on an aluminum component attached to the outside of the water so that heat can transfer. In other words, always have a way to connect heated parts to the outside water physically.

- Convection

  - Pickups (secondary option) - bring in outside water, and a coil wraps around all the heated components so that water can transfer through the internals and cool the components.

  - A fan moves fluid (air) to increase heat transfer. This would be running pipe and watercooling components, but this increases complexity and points of failure, so we should opt for conduction as the first choice.

- Radiation

  - Energy gets trapped in an enclosed space, increasing heat over time.

Motor architecture

- Dual Outboard motor

  - High maneuverability

  - More expensive

  - Less likely to interfere with electrical components

  - This is the way last year's winner was designed, probably the best choice

- In Board

  - One motor supplies power to multiple components

  - Generates a ton of heat and vibration, and EMS

Waterproofing

- Buy waterproof motors

- Any tubing we introduce for cooling will create more points of failure and will introduce contact resistance. Best to rely on physical conduction.

- Fittings should be on the outside of the robot and machined into the robot so there are no seams or gaps for leakage. This is as opposed to a threaded insert for tubes that run across components.

- Follow other teams to see what components they use to waterproof exposed areas.

Neutral Buoyancy

- Option

  - Compress and decompress air to increase or decrease buoyancy so that the robot can rise and fall (gas piston system). Can probably find this exact system online for purchase because they are used a lot in RC.

- Option

  - Try to keep buoyancy as neutral as possible, and a weight can be adjusted to change the angle of propulsion.

- Both are solutions to decrease the number of motors we need. If we have money for lots of motors, we can add a bunch for changing direction, pitch, or elevation.