Mechanical Advantage = MA

What is mechanical advantage in gears? | igus® Engineer's Toolbox

MA - Amplification of Force/Speed

- MA = Output Force/Input Force

  - If an output gear has 40 teeth, and input gear has 10 teeth, MA = 4

  - Higher Gear Ratio equals Greater Force, but reduced Speed

    - Torque Ratio >1

    - Speed Ratio <1

Choosing Gears & Gear Ratios for Robots: A Beginner-Friendly Guide – ThinkRobotics.com

Gear Types:

- Spur Gears - Ideal for Parallel Shafts

- Bevel Gears - Used to Change Axis of Rotation

- Worm Gears - High Torque and Compact Design, used in lifting Mechanisms

- Planetary Gears - Compact and efficient systems in high torque, low speed applications like robotic wheels

- Helical Gears - Quieter and Smoother than spur gears due to angled teeth,

Choosing Gear Ratio:

- Determine Characteristics of Motor

  - RPM - Can be Geared down for Torque

  - Torque Rating - Can amplify Torque

- Function

  - Robotic Arms - Need more torque for lifting, ratios of 1:20 or higher is common

Key Performance Parameters of Blue Robotics Claw:

Newton Subsea Gripper for the BlueROV2 and other ROVs

| Grip Force (Tip) | 97 N |
| --- | --- |
| Grip Force (Middle) | 124 N |
| Push-Rod Axial Load Rating | 3 Kg |
| Time to Open/Close | 1.6 Seconds |

Push-Rod Axial Load Ratings

Effective Weight = mg(1 - Water Density/Object Density)

Force of Gripper = mg/2(meu)

- F Required = Factor of Safety(Above 4?) * Force of Gripper

- Joint Lift Capability?

Geargenerator.com