Size Test:
Requirement: The ROV must be able to fit through a 1 by 1 meter square hole without obstruction.

Procedure:

- Measure the L/W/H

Acceptance Criteria:

- Any combination of two dimensions both being under 1 meter

Weight Test:
Requirement: The ROV's dry mass must be under 35 kg.

Procedure:

- Measure the L/W/H

Acceptance Criteria:

Any combination of two dimensions both being under 1 meter

Water Immersion Test:

Requirement: The float materials shall experience 0 physical or chemical changes in saltwater or salt brine pool environments for the length of the project timeline.

Procedure:

- Submerge the bare chassis for 30 minutes in a representative environment.

- Remove and inspect immediately

Acceptance Criteria:

- No Corrosion

- No Deformation

- No Trapped Air

Static Load Test:

Requirement: The float materials shall experience 0 physical or chemical changes in saltwater or salt brine pool environments for the length of the project timeline.

Procedure:

- Apply Downward load equal to 2x vehicle dry weight

- Hold load for 60 seconds

Acceptance Criteria:

- No permanent deformation

- No fastener loosening

- No visible cracking or yielding

Vibration & Fatigue Check:

Requirement: The float materials shall experience 0 physical or chemical changes in saltwater or salt brine pool environments for the length of the project timeline.

Procedure:

- Run thrusters at 50-70% power for 10 minutes.

- Observe joints, mounts, and frames intersections

Acceptance Criteria:

- No deformation

- No fastener back-out

Water Proofing Test:

Requirement: The ROV shall have 0 water leaks during submersion.

Procedure:

- Place Napkins/Water absorbent material inside the electronics enclosure.

- Submerge the electronic enclosure for 30 minutes.

- Remove and inspect immediately

Acceptance Criteria:

- Water absorbent material hasn’t absorbed water

Thermal Regulation Test:

Requirement: The ROV's electronics shall maintain temperatures between 0 C and 35 C.

Procedure:

- Allow electronics to reach a resting internal temperature that is representative of the competition

- Run electronics in a manner that is representative of operation (power draw) with representative outside environment (temperature)

- Do this for a duration of time that is the same as the competition

- Run for multiple cycles to ensure reliability

Acceptance Criteria:

- Enclosure does not fall out of temperature range

- Enclosure does not drop in performance over multiple cycles