Project Plan - TMS

Goals: Create a 3 Week Goal list with Measureables and Deliverables

| Goal | Measureables | Deliverables |
| --- | --- | --- |
| Finish CAD for Chassis And check new material (PLA) | - Completion of frame - Plan for 3D printing so striations are strong and won’t result in cracking - More possible fluid tests in water - Make sure PLA is insoluble - 3D print Tapered Washers | - Completed Assembly in GitHub - Bill and plan for filament - Assembly is spliced and ready for 3D printing - Tapered Washers Cadded |
| Float | - Collaboration and deliberate new design options for float - Create solution for creating an internal ballast system |  |
| Improvements on claw | - Complete Deliberation about claw design and whether we want changes or improvements. | - Final Idea or improvement or changes - Changes are implemented |

Team Members:

| Name | Skills | Training Focus |
| --- | --- | --- |
| Alexandria Llavona | SolidWorks, Manufacturing, | Leading the Sub-Team to Create a Functional ROV |
| Matthew | Cad and prototyping Experience | TBD |
| Ryan | Cad and Stress Work | TBD (CAD) Stress testing* |
| Elif | Cad and Manufacturing Experience | TBD |

| Software/Tool | Purpose | Members | Resources/Links |
| --- | --- | --- | --- |
| SolidWorks | 3D Modeling | All Members |  |
| Fab lab/RPS/FEDC | Constructing and Printing ROV | TBD |  |

Required Softwares:

Training Plan:

- Required Tools/Software

  - Get Red Badge!

    - Go to the FEDC website and sign up for the red badge training as soon as possible please!

  - Team members will be able to get familiar with Solidworks and how to Cad, Print, and build the final product.

Documentation:

- Chassis Issues for Future

  - CAD: Thicken lower and upper enclosure so heated inserts don’t pop through

  - Striation Cracks

    - Long-Term Fix: Print the Chassis parts long-ways (cracks form along the striations)

    - Short Term Fix: Heat Gun, set to 305*C, heat til plastic melts together

- The ballast system is currently external, and needs to be internal!

- Limit the holes on the float, use multiple methods to waterproof

- Claw Parts

  - Wrist Joint good, Holes in Claw joint need remodeling (too close to edge)

- Thicken middle section due to heated inserts sticking through

- Use heated inserts! Much better than tapping.

  - If heated insert starts to twist (good practice is leaving the screw we wanna use for that hole in the heated insert hole)

    - Heat the soldering iron, remove it, and use a 3D pin to fill the hole, redrill it, and place a new heated insert

  - If the screw is having difficulty passing through the insert and won't move, there is most likely plastic in the threads of the heated insert.

    - You can either pick out the threads (super time consuming), or repeat the process of placing a new insert

- Brackets

  - Need to be bigger, too thin and flex too easily

  - Any metal going on the claw should use the metal I bought this semester (it is thicker and more difficult to break.

  - Issues with holes and brackets not aligning

    - Fix: You can drill the bracket hole to be larger, but good practice for this semester would be to drill all the holes first and place the heated inserts (making sure screws go in) and measure the distance with calipers.

- Have Better Naming Conventions 🤭

- Use Zip Files!

Timeline:

SUMMER (3 Weeks Plan)

- Create teams for each component of the TMS Project, Time:

- Get changes made to the chassis , Time:

- Deliberate changes to float (get the ballast system within the float)

- Have all new copies of the Chassis saved in our Github!

  - https://github.com/3lover/TAMU-MateROV

- Discuss and make modifications to the claw, Time:

- Have Red Badge Training Completed

FALL SEMESTER

- After first PDR, Start 3D printing PLA Chassis Parts Before Rush of 3D printing Begins

- Print new Brackets

- Assemble Chassis and Thrusters

  - This should be done as early as possible so we can hand the ROV Chassis to GNC so they can start testing their code

  - We can also place GNC sensor during this time

- Once Competition comes out, we can print claw parts

- Construct Claw parts (this may align with finals so this is okay to be done next semester)

SPRING SEMESTER

- Assemble claw with ROV, test it, and if there are no immediate changes or problems, hand it over to GNC for their code-testing.