# ROV Assembly

# Mechanical ROV

3D printing procedures:

– RPS

–FEDC

Bassplate

- Sliced into 4 pieces

6.35 mm screws (¼ inch)

6.35 mm nuts

Required:

4 x Aluminum 6061 brackets -> Laser cut

Chassis

- Chassis + Bassplate

Required:

4 x Aluminum 6061 brackets -> Laser cut

Top side

M3-18mm screws

M3 heated inserts

Bottom side

6.35 mm screws (¼ inch)

6.35 mm nuts

Electronics Enclosure Platform:

- Slice the Electronics Enclosure Platform and utilize the PLA

  - ROV Electronics Tray for 4" Tube

Polyurethane Potting:

- 2 part potting compound, 2:1 mix ratio

- 5-7 minute working time

- Pour into component

- 24 hour cure at room temperature

- Constant service temperature of -50 to 120 °C (-58 to 248 °F)

Thrusters

- Mesh

  - 3D printed using PETG

  - Attached to thrusters on both sides using a clamping mechanism (or using other methods such as epoxy, superglue, etc.)

- Thrusters

  - Will need to use plates (vertical) or brackets (horizontal) to mount the thrusters

  - 80 Heat set inserts in total are needed to mount the thrusters

    - The heat set inserts will be testing on a singular mount before attaching to the chassis to make sure it provides a secure connection. The length provides 2mm of secure fit at the base, and the inserts are applied through the back and are multidirectional.

  - M4 screws are provided with the thrusters, and extra screws were purchased.

# Float Assembly

# Mechanical

Hull:

- Drill holes in the hull for two sensors on the end cap and two openings for the ballast pump. Total of 4 holes.

  - Ultrasonic sensor hole dimensions:

  - Depth sensor hole dimensions:

  - Ballast hole dimensions:

- Drill small holes (not through) into the top end cap

  - Hole dimensions:

- Epoxy the recovery feature into the end cap holes

- Cut foam into a cylindrical shape, drill holes, then hollow out the inside. (alternatively, cut foam flat to the correct thickness, drill holes, then form it to the inside of the hull).

  - Foam thickness:

  - Hole dimensions same diameters as hull holes

- Epoxy zinc weight to the bottom end-cap

- Epoxy electronics harness to the top end-cap

- Polyurethane-pot the sensors into the end-cap holes

- Apply the heating pad to the underside of the electronics harness

Zinc weight procedures:

- Weigh total system mass (expecting to be slightly too heavy), then drill small holes into the zinc until we are exactly neutrally buoyant.

Electronics harness procedures:

- Lathe diameter to size

- Mill side openings

- Lathe inner diameter to size

- Saw off excess length

- Face the sawed-off side using the lathe facing tool

Ballast:

- Peristaltic Pump will be installed within the float enclosure

- Holes for the intake and exhaust tubes will need to be drilled into the enclosure

  - Waterproofing will be needed to the tubes

- Exhaust tube will be connected to the bag, intake tube will be secured to the bottom of the pump

  - Water bag will be secured to the bottom of the float

    - Will need to either print a bracket to hold, zip tie the bag, or glue to to the hull

# Electrical

# Ground Station Assembly