| Guideline | Page |
| --- | --- |
| Wiring discipline/workmanship (ELEC-023P) | 43 |
| Strain relief at both ends of the tether (ELEC-024P) | 43 |
| Safety In Design (SID) technical documentation does not show a main fuse | 43 |
| Circuit boards, batteries, and components must be securely fastened | 43 |
| Full Load Amps (FLA) must be included on the SID | 47 |
| All electrical devices must be mapped on the SID | 48 |
| If using 48-volts, an SBS50 Anderson Powerpole connector is the main point of connection to the MATE supply. If using 12-volts, a red-black Anderson powerpole connection is the main point of connection to the MATE supply (ELEC-010P) | 48 |
| A properly sized fuse is within 30 cm of the main point of connection to power. If the company is using 48-volts, a properly sized Littelfuse must be used. If the company is using 12-volts, a properly sized ATO blade or Mini blade fuse must be used. The company must use a ruler to show this distance (ELEC-008P) | 48 |
| Full load amps value and fuse size must be selected (ELEC-008P) | 49 |
| The inside of the control box must not have exposed wiring (ELEC-017P) | 49 |
| The control box must be neatly laid out with attention to workmanship (ELEC-022P) and a separation and identification of 120VAC wiring from DC and control voltages (ELEC-023P) | 49 |
| The tether leading to the control system and the ROV must have adequate strain relief (ELEC-024) | 49 |
| The tether must be capable of reaching 5.5m down and 8m out | 53 |
| Only one of 48V or 12V may be chosen for the power supply | 54 |
| All power provided externally must be from the power supply and pass through the provided fuse and the in-line fuse | 54 |
| The ROV must use a specified in-line fuse (linked to in the guidelines), and the specified fuse holder if using 48V (for 12V, different fuses are required) | 54 |
| All blade fuses must correspond to the standardized color codes | 54 |
| The 48V power supply output may not be modified before reaching the ROV | 55 |
| Voltage can’t be increased above the nominal voltage anywhere on the ROV | 55 |
| Only specific types of batteries allowed (see guidelines for details if needed) | 56 |
| Battery fuses and wiring is restricted (see guidelines for details if needed) | 57 |
| The enclosure must open if the inside pressure is greater than outside | 58 |
| Pressure release valves and pop off end caps are not allowed | 59 |
| Independent sensors must have a 3 amp fastblow fuse on the positive side | 59 |
| The 48V is limited to 30A, and the 12V is limited to 25A | 59 |
| Use the provided tables for deciding on a fuse size | 60 |
| The fuse must be connected before any E-stops or other safety systems | 61 |
| For 48V, power supply connections will be Anderson Power Connectors, which must be securely connected to the ROV power wires | 62 |
| The Anderson SBS50 connector is required (linked in guidelines) | 62 |
| Tether must be of IEC standard type, and have a high and low voltage | 63 |
| Electrical connections must be sealed from water | 64 |
| If disconnected from the surface supply, the ROV must shut down in 5s | 65 |
| All circuit board connections should be soldered or through terminal connections with screw or spring locks. Pin jumpers should not be used in the final system and will not pass safety inspection | 1 (EWP) |

Sources:

Mate ROV 2025 Guidelines

https://20693798.fs1.hubspotusercontent-na1.net/hubfs/20693798/2025%20PIONEER%20Manual_FINAL_1_6_withCover.pdf

Mate ROV Expected Work Practices

https://20693798.fs1.hubspotusercontent-na1.net/hubfs/20693798/MATE%20ROV%20Competition%20Tech%20Bulletin%20-%20MATE%20Expected%20Work%20Practices.pdf